205-VID-018 HD Camera

Unit Test Specification — Rev { 1.01 }

| Tester: | _________________________ | Date: |  |
| --- | --- | --- | --- |
| Unit S/N: | _________________________ | SW Rev: | 1.01 |

| ☐ | Test | Result | Notes |
| --- | --- | --- | --- |
| TC-EE — EEPROM Emulation |  |  |  |
| ☐ | TC-EE-01: Init formats blank flash EE_Init on completely blank flash (both pages erased) | ☐ Pass ☐ Fail ☐ Skip |  |
| ☐ | TC-EE-02: Read with no valid page returns no valid page Reading before EE_Init (no valid page exists) must return NO_VALID_PAGE. | ☐ Pass ☐ Fail ☐ Skip |  |
| ☐ | TC-EE-03: Read absent variable returns not found Reading a virtual address that was never written returns 1 (not found). | ☐ Pass ☐ Fail ☐ Skip |  |
| ☐ | TC-EE-04: Write read roundtrip Writing a variable then reading it back must return the same value. | ☐ Pass ☐ Fail ☐ Skip |  |
| ☐ | TC-EE-05: Multiple variables roundtrip Three distinct virtual addresses can coexist in the same page. | ☐ Pass ☐ Fail ☐ Skip |  |
| ☐ | TC-EE-06: Overwrite returns latest value Writing the same virtual address twice must return the most recent value. | ☐ Pass ☐ Fail ☐ Skip |  |
| TC-MV — Menu Variables |  |  |  |
| ☐ | TC-MV-01: Incvar increments numeric value incvar on a numeric variable increments value by inc. | ☐ Pass ☐ Fail ☐ Skip |  |
| ☐ | TC-MV-02: Incvar wraps numeric at max incvar at max wraps around to min. | ☐ Pass ☐ Fail ☐ Skip |  |
| ☐ | TC-MV-03: Decvar decrements numeric value decvar on a numeric variable decrements value by inc. | ☐ Pass ☐ Fail ☐ Skip |  |
| ☐ | TC-MV-04: Decvar wraps numeric at min decvar at min wraps around to max. | ☐ Pass ☐ Fail ☐ Skip |  |
| ☐ | TC-MV-05: Fast inc jumps by ten When fast_inc is set, incvar jumps by inc × 10. | ☐ Pass ☐ Fail ☐ Skip |  |
| ☐ | TC-MV-06: Getvalue finds first enum getvalue() with an enum list and str_value matching the FIRST entry | ☐ Pass ☐ Fail ☐ Skip |  |
| ☐ | TC-MV-07: Getvalue finds middle enum getvalue() with str_value matching the MIDDLE entry sets value to 2. | ☐ Pass ☐ Fail ☐ Skip |  |
| ☐ | TC-MV-08: Getvalue finds end enum getvalue() with str_value matching the END entry sets value to 3. | ☐ Pass ☐ Fail ☐ Skip |  |
| ☐ | TC-MV-09: Incvar advances enum incvar on an enum variable at entry 1 ("OFF") advances to entry 2 ("ON"). | ☐ Pass ☐ Fail ☐ Skip |  |
| ☐ | TC-MV-10: Incvar wraps enum at last entry incvar at the last enum entry wraps back to entry 1 (min = 1). | ☐ Pass ☐ Fail ☐ Skip |  |
| ☐ | TC-MV-11: Decvar wraps enum at first entry decvar at the first enum entry (value = min = 1) wraps to the last | ☐ Pass ☐ Fail ☐ Skip |  |
| ☐ | TC-MV-12: Getstrval formats integer getstrval on a numeric integer variable right-aligns value within | ☐ Pass ☐ Fail ☐ Skip |  |
| ☐ | TC-MV-13: Getstrval formats enum getstrval on an enum variable selects the token at value (1-based) | ☐ Pass ☐ Fail ☐ Skip |  |
| ☐ | TC-MV-14: UpdateVarByValue sets value and str UpdateVarByValue sets value and refreshes str_value via getstrval. | ☐ Pass ☐ Fail ☐ Skip |  |
| ☐ | TC-MV-15: UpdateVarByString numeric updates value UpdateVarByString on a numeric variable parses the string to a | ☐ Pass ☐ Fail ☐ Skip |  |
| ☐ | TC-MV-16: UpdateVarByString enum updates value UpdateVarByString on an enum variable scans the token list and sets | ☐ Pass ☐ Fail ☐ Skip |  |
| TC-SB — Save/Load Subroutines |  |  |  |
| ☐ | TC-SB-01: Save load variables micOnOff Save_Variables packs all menu str_values into EEPROM. | ☐ Pass ☐ Fail ☐ Skip |  |
| ☐ | TC-SB-02: Save load variables nullvar Load_Variables restores the first struct in the .menu_data section | ☐ Pass ☐ Fail ☐ Skip |  |
| ☐ | TC-SB-03: Save load serial num round trip Save_Serial_Num writes each character of SerialNum.str_value to a | ☐ Pass ☐ Fail ☐ Skip |  |
| ☐ | TC-SB-04: Save serial num skips when already set Save_Serial_Num is a no-op when EEDataTable already holds a numeric | ☐ Pass ☐ Fail ☐ Skip |  |

Overall Result:  ☐ PASS   ☐ FAIL          Signature: _______________________   Date: ____________